NTF201 – Sample Midterm Examination (Version A)

Networking Fundamentals Instructor

2026-02-01

Introduction to Computer Networks
Time: 2 Hours | Total Marks: 120

Part A – Matching (10 Marks)

Instructions: Match each item in Column A with the correct answer in Column B. Write the letter of your choice in the Answer column.

| No. | Column A | Answer | Column B |
| --- | --- | --- | --- |
| 1. | Protocol that translates domain names to IP addresses | _____ | A. CSMA/CD |
| 2. | Maximum transmission unit for Ethernet frames | _____ | B. DNS |
| 3. | Collision detection method used in Ethernet | _____ | C. 1500 bytes |
| 4. | Layer responsible for end-to-end communication | _____ | D. Router |
| 5. | Device that forwards packets between networks | _____ | E. Transport Layer |
| 6. | Protocol used for reliable data transmission | _____ | F. MAC Address |
| 7. | Unique identifier for network interface cards | _____ | G. TCP |
| 8. | Process of dividing networks into smaller subnets | _____ | H. Subnetting |
| 9. | Protocol for dynamic IP address assignment | _____ | I. DHCP |
| 10. | Standard for wireless local area networks | _____ | J. IEEE 802.11 |

Part B – Short Answer (30 Marks)

Instructions: Answer each question in 2-3 sentences. Each question is worth 3 marks.

- Explain the difference between connection-oriented and connectionless protocols. Provide examples of each.

- What is the purpose of the subnet mask in IP addressing? How does it help in network communication?

- Describe the function of ARP (Address Resolution Protocol) and when it is used in network communication.

- What are the main differences between hubs, switches, and routers in terms of functionality?

- Explain the concept of collision domains and broadcast domains in Ethernet networks.

- What is the difference between half-duplex and full-duplex communication? Which is preferred and why?

- Describe the purpose and functionality of NAT (Network Address Translation).

- What is VLAN (Virtual Local Area Network) and what are its main advantages?

- Explain the difference between static and dynamic routing. When would you use each approach?

- What is the purpose of the Time-to-Live (TTL) field in IP packets?

Part C – Multiple Choice Questions (40 Marks)

Instructions: Choose the best answer for each question. Each question is worth 2 marks.

EASY QUESTIONS (50% - 10 questions)

- Which layer of the OSI model is responsible for routing packets?

  - A. Physical Layer

  - B. Data Link Layer

  - C. Network Layer

  - D. Transport Layer

- What is the default subnet mask for a Class C network?

  - A. 255.0.0.0

  - B. 255.255.0.0

  - C. 255.255.255.0

  - D. 255.255.255.255

- Which protocol operates at the Transport Layer?

  - A. IP

  - B. Ethernet

  - C. TCP

  - D. ARP

- What is the maximum number of hosts in a /24 network?

  - A. 254

  - B. 255

  - C. 256

  - D. 512

- Which device operates at the Physical Layer?

  - A. Router

  - B. Switch

  - C. Hub

  - D. Bridge

- What is the standard port number for HTTP?

  - A. 21

  - B. 23

  - C. 80

  - D. 443

- Which topology connects all devices to a central hub?

  - A. Bus

  - B. Ring

  - C. Star

  - D. Mesh

- What does CSMA/CD stand for?

  - A. Carrier Sense Multiple Access/Collision Detection

  - B. Circuit Switched Multiple Access/Collision Domain

  - C. Carrier Signal Multiple Access/Connection Detection

  - D. Circuit Signal Multiple Access/Connection Domain

- Which cable type is most commonly used for Ethernet connections?

  - A. Coaxial

  - B. Fiber optic

  - C. Twisted pair

  - D. Serial

- What is the purpose of a default gateway?

  - A. To assign IP addresses

  - B. To resolve domain names

  - C. To route traffic to other networks

  - D. To detect collisions

MEDIUM QUESTIONS (20% - 4 questions)

- In a switched network, what happens when a switch receives a frame with an unknown destination MAC address?

  - A. Drops the frame

  - B. Floods the frame to all ports except the source

  - C. Sends an error message

  - D. Forwards to the default gateway

- Which routing protocol uses hop count as its metric?

  - A. OSPF

  - B. BGP

  - C. RIP

  - D. EIGRP

- What is the main advantage of using switches over hubs?

  - A. Lower cost

  - B. Easier configuration

  - C. Each port has its own collision domain

  - D. Support for wireless connections

- In subnetting, what does a /26 network provide?

  - A. 62 host addresses

  - B. 64 host addresses

  - C. 126 host addresses

  - D. 254 host addresses

HARD QUESTIONS (30% - 6 questions)

- Which statement about VLSM (Variable Length Subnet Masking) is correct?

  - A. All subnets must use the same subnet mask

  - B. It allows efficient use of IP address space

  - C. It only works with classful routing protocols

  - D. It requires manual IP address assignment

- In the TCP three-way handshake, what is the correct sequence?

  - A. SYN, ACK, SYN-ACK

  - B. SYN, SYN-ACK, ACK

  - C. ACK, SYN, SYN-ACK

  - D. SYN-ACK, SYN, ACK

- What is the primary difference between distance vector and link state routing protocols?

  - A. Distance vector uses bandwidth as metric

  - B. Link state protocols converge faster

  - C. Distance vector protocols are more secure

  - D. Link state protocols use less memory

- Which mechanism does TCP use for flow control?

  - A. Congestion window

  - B. Sliding window

  - C. Time window

  - D. Buffer window

- In a network using STP (Spanning Tree Protocol), what is the purpose of blocking ports?

  - A. To increase security

  - B. To prevent routing loops

  - C. To prevent switching loops

  - D. To save bandwidth

- What happens when the TTL field in an IP packet reaches zero?

  - A. The packet is forwarded normally

  - B. The packet is discarded and an ICMP message is sent

  - C. The packet is sent to the default gateway

  - D. The TTL is reset to its original value

Part D – Network Design Questions (20 Marks)

Instructions: Answer all questions. Show your work clearly and draw diagrams where required.

- [5 marks] Design a network topology for a small office with the following requirements:

  - 3 departments: Sales (15 computers), Engineering (20 computers), Administration (8 computers)

  - Each department should be on a separate subnet

  - Include internet connectivity

  - Show all network devices and connections

  - Specify IP addressing scheme using 192.168.1.0/24

- [4 marks] Analyze the following network scenario and complete the missing components:

  - Given Network Setup:

    - Main office: 192.168.10.0/24

    - Branch office: 192.168.20.0/24

    - WAN connection between offices

  - Requirements: a) Design the complete network showing routing between offices b) Configure appropriate routing protocols c) Include redundancy for critical connections d) Specify VLAN configuration if needed

- [4 marks] Convert the following business requirements into a detailed network design:

  - Corporate Network Requirements:

    - 200 employees across 4 floors

    - Guest wireless network (isolated from corporate network)

    - Server farm with web, email, and database servers

    - Remote access capability for employees

    - High availability and security requirements

    - Internet connectivity with backup ISP

- [4 marks] Identify and correct the issues in the following network configuration:

  - Problematic Network Setup:

    - All devices on single subnet 192.168.1.0/24

    - Single hub connecting 50 computers

    - No VLANs configured

    - Single point of failure for internet connection

  - Questions: a) Identify performance and security issues (2 marks) b) Suggest improvements to the network design (2 marks)

- [3 marks] Draw network diagrams and explain the following concepts:

  - Star topology with central switch

  - VLAN configuration across multiple switches

  - Router-on-a-stick configuration

  - Redundant links with STP

  - Wireless access point placement

Part E – Subnetting and Configuration Questions (20 Marks)

Instructions: Show all steps in your subnetting calculations with clear explanations.

- [6 marks] Subnet the network 172.16.0.0/16 to meet the following requirements:

  - Subnet Requirements:

    - Subnet A: 1000 hosts

    - Subnet B: 500 hosts

    - Subnet C: 250 hosts

    - Subnet D: 100 hosts

  - Show the following for each subnet:

    - Subnet address

    - Subnet mask

    - First usable host address

    - Last usable host address

    - Broadcast address

- [5 marks] Complete the subnetting process for the following scenario:

  - Given: Network 10.0.0.0/8 needs to be divided into 16 equal subnets

  - Requirements: a) Calculate the new subnet mask (1 mark) b) List the first 8 subnet addresses (2 marks) c) For subnet 10.64.0.0, provide host range (1 mark) d) Calculate total number of hosts per subnet (1 mark)

- [4 marks] Analyze the following IP configuration and identify issues:

  - Network Configuration:

    - Device A: 192.168.1.100/25, Gateway: 192.168.1.1

    - Device B: 192.168.1.200/25, Gateway: 192.168.1.1

    - Device C: 192.168.1.50/24, Gateway: 192.168.1.1

  - Show step-by-step analysis:

    - Which devices can communicate directly?

    - Identify configuration errors

    - Suggest corrections

- [3 marks] Based on the following requirements, create an IP addressing plan:

  - Campus Network Requirements:

    - Building A: 300 devices

    - Building B: 150 devices

    - Building C: 75 devices

    - Management network: 20 devices

    - Use private IP addressing starting with 10.0.0.0/8

  - Create the addressing plan with subnet details

- [2 marks] Explain VLSM benefits and provide a practical example:

  - VLSM advantages:

  - Practical scenario where VLSM is beneficial:

  - Comparison with fixed-length subnetting:

END OF EXAMINATION

Note: This is a sample examination for practice purposes. Ensure all answers demonstrate clear understanding of networking concepts. Partial marks will be awarded for correct approaches and methodology. Good luck with your studies!